# Single Chlorine Room Door TT2.1/RC3

## Access door specifically designed for buildings in drinking water supply systems where chlorine gas may be generated

## RC3 doors are suitable for buildings with an increased risk of burglary where a higher level of security is required.

Chlorine room door, execution in accordance with DIN 19606, burglar and attack proof, certified to security of DIN EN 1627, resistance class RC3, ready for installation, single-leaf, double skinned, completely made from 1.4404 stainless steel (AISI 316 L), with double rubber seal, the door and frame closing flush, the door opening to the outside (in escape direction). Sound insulation: 34/29 dB.

Door leaf with injected CFCs-free polyurethane hard foam insulation, longitudinally ground surface finish, door leaf inner side painted in stainless steel look (RAL 9006), with rubber seal. Fastened to the frame with stable maintenance-free hinge plates. The closed door engaging flush with the frame by means of solid anchor bolts engaging in the frame for four-point locking. Surface insert and drill protection plate in the lock area for the anti-panic function

Galvanized safety lock with stainless steel forend, with austenitic manganese steel plate as anti-drilling protection, provided for a customer-provided mechanical profile half cylinder in accordance with DIN 18252:2006- 12,21-,31-,71-BZ), with anti-panic lock, i.e. the door canany time be opened from inside by operating the handle, even when locked. Protective stainless steel fitting, without cylinder cover, with a rigidly mounted turning handle and a red plastic handle inside.

Frame, three-sided, fabricated from special section, with inserted rubber seal, with step flashing, including fixing material.

Frame and door shielded arc-welded and acid bath cleaned before washing, drying and surface passivation.

Ordering options:

Frame / mounting:

➤ Z-frame – bolted with dowelling clips

➤ Block frame – bolted with dowelling clips

➤ Block frame – bolted through the frame

## Door opening options:

➤ Hinges on left

➤ Hinges on right

## Options:

➤ Prepared for a digital profile cylinder to be provided by the customer

➤ Lockable from inside with a second handle

➤ Lock for two side-by-side profile cylinders (that can be opened independently)

➤ Stainless steel rain protection rosette for the stainless steel profile cylinder

➤ Upper door closer, lockable in position

➤  Two-piece floor door arrestor/stop

➤  One-piece door arrestor

➤ Upper door stopper can be released manually

➤ Block frame insulation

➤ Lowerable floor seal

➤ Magnetic contact: “door open” or “door closed” status display

➤ Lock-bolt contact: “locked” or “unlocked” status display

➤ Potential equalisation on frame

➤  Rain rail on the top of the frame

➤ Surface finish (lacquer with or without anti-graffiti properties, copper covering, wooden panelling)

➤ Steel sheet finish for exterior door surface for installations exposed to sunlight

Note:

## If the door is exposed to sunlight, its operability may be affected. In such a case, it is recommended to equip the door outside with a wood panelling or steel sheet.